U.S. CUSTOMS AND BORDER PROTECTION

LABORATORY METHODS

CBPL METHOD 15-10

# High Performance Liquid Chromatography Determination of

# Identity of Oils and Their Mixtures:  Isocratic Elution

SAFETY STATEMENT

This CBPL Method cannot fully address safety issues that may arise from its use.  The analyst is responsible for assessing potential safety issues associated with a given method at its point of use.

Before using this method, the analyst will consider all general laboratory safety precautions.  In particular, the analyst will identify and implement suitable health and safety measures and will comply with all pertinent regulations.

METHOD UNCERTAINTY

The uncertainty of measurement for this method is specific to each laboratory.

0.		INTRODUCTION

The identification of specific vegetable oils and mixtures thereof is conveniently first approached by performing a High Performance Liquid Chromatography (HPLC) separation of the component triacylglycerides (TAGs).  The HPLC procedures are (a.) isocratic elution (constant solvent composition) and (b.) gradient elution (varying solvent composition with elution time).  This method addresses isocratic elution with a refractive index (RI), ultraviolet (UV), or evaporative light scattering (ELSD) detector.

1.		SCOPE AND FIELD OF 					APPLICATION

This method is a standard American Oil Chemists’ Society (AOCS) procedure that is applicable to all varieties of vegetable oils and their mixtures, which are listed in Chapter 15 of the Harmonized Tariff Schedules of the United States (HTSUS).  References 2.4 and 2.5 are listings to direct the reader to the choice of operating conditions for specific oils and the appropriate HPLC chromatograms for identification purposes.

2.		REFERENCES

2.1		AOCS Official Method Ce 5b.  “Triglycerides in Vegetable Oils by HPLC.”  Note that this method achieves identification of the TAGs by calculating Partition Numbers for the individual TAGs.  However, the chromatograms can be used directly for identification of the oils.

2.2		AOCS Official Method Ce 5c.  “Individual Triglycerides in Oils and Fats by HPLC.”  Note that this method achieves identification of the TAGs by addition of internal standard TAGs.  Quantitation of the other TAGs is based on the internal standards.  However, the chromatograms can be used directly for identification of oils.

2.3		CBPL Method 15-11.  “High Performance Liquid Chromatographic Determination of Identity of Oils and Their Mixtures:  Gradient Elution.”

2.4		Flor, R.V. and Tiet, L.T.  “HPLC Identification of Common Vegetable Oils on a Five Micron Octadecyl (C-18) Column.”  USCS Technical Bulletin.  20(2).  Pp. 40-63.  1986.

For Corn, Cottonseed, Coconut, Olive, Palm Kernel, Peanut, Safflower, Soybean, Sunflower Seed, and Rapeseed (Canola) oils.

2.5		Flor, R.V., Hecking, L.T., and Martin, B.D.  “Development of HPLC Criteria for Determination of Grades of Commercial Olive Oils.  Part 1.  The Normal Ranges for the Triacylglycerols.”  J. Amer. Oil Chem. Society.  70(2).  Pp. 199-203.  1993. (Olive oils.)

END