U.S. CUSTOMS AND BORDER PROTECTION

LABORATORY METHODS

CBPL METHOD 27-02

ASTM D 1298

Standard Test Method for Density, Relative Density (Specific

Gravity), or API Gravity of Crude Petroleum and

Liquid Petroleum Products by Hydrometer Meter

SAFETY STATEMENT

This CBPL Method cannot fully address safety issues that may arise from its use.  The analyst is responsible for assessing potential safety issues associated with a given method at its point of use.

Before using this method, the analyst will consider all general laboratory safety precautions.  In particular, the analyst will identify and implement suitable health and safety measures and will comply with all pertinent regulations.

METHOD UNCERTAINTY

The uncertainty of measurement for this method is specific to each laboratory.

0.		SCOPE AND FIELD OF 					APPLICATION

This method covers the laboratory determination, using a glass hydrometer, of the density, relative density (specific gravity), or API gravity of crude petroleum, petroleum products, or mixtures of petroleum and non-petroleum products normally handled as liquids, and having a Reid vapor pressure 26 psi or less.  Values are measured on a hydrometer at temperatures, readings of density being reduced to 15 oC, and readings of relative density (specific gravity) and API gravity to 60 oC, by means of international standard tables.  This method is applicable for classification of crude petroleum oils and crude oils obtained from bituminous minerals (e.g., from shale, calcareous rock, and sand).

Crude petroleum oils and crude oils are classifiable according to their API gravity in the Harmonized Tariff Schedule of the United States (HTSUS) Chapter 27, Heading 2709.  Accurate determination of the gravity of crude petroleum oils and crude oils is necessary for the conversion of measured volumes at a measured temperature to volumes at standard temperature of 60 oF.  API gravity also is an important factor for determining the quality of crude oils.

1.		REFERENCE

1.1		ASTM D 1298.  “Standard Test Method for Density, Relative Density (Specific Gravity), or API Gravity of Crude Petroleum and Liquid Petroleum Products by Hydrometer Method.”

END