U.S. CUSTOMS AND BORDER PROTECTION

LABORATORY METHODS

CBPL METHOD 27-10

ASTM D 323

Standard Test Method for Vapor Pressure

of Petroleum Products (Reid Method)

SAFETY STATEMENT

This CBPL Method cannot fully address safety issues that may arise from its use.  The analyst is responsible for assessing potential safety issues associated with a given method at its point of use.

Before using this method, the analyst will consider all general laboratory safety precautions.  In particular, the analyst will identify and implement suitable health and safety measures and will comply with all pertinent regulations.

METHOD UNCERTAINTY

The uncertainty of measurement for this method is specific to each laboratory.

0.		SCOPE AND FIELD OF					APPLICATION

This method is applicable to commodities provided for in Chapter 27 of the Harmonized Tariff Schedule of the United States (HTSUS). It covers the determination of the vapor pressure of gasoline, volatile crude oil, and other volatile petroleum products.

This method consists of four procedures based on the expected vapor pressure of the petroleum product.  Procedure A is applicable to gasoline and other petroleum products with a vapor pressure of less than 180 kPa.  Procedure B may also be applicable to these other materials, but only gasoline was included in the interlaboratory test program to determine the precision of this test method.  Neither procedure is applicable to liquefied petroleum gases or fuels containing oxygenated compounds other than methyl t-butyl ether (MTBE).   Procedure C is for materials with a vapor pressure greater than 180 kPa.  Procedure D is for aviation gasoline with a vapor pressure of approximately 50 kPa.  This method is applicable for determining volatility characteristics of automotive spark-ignition engine fuels, per 1.2, such as gasolines, and specific gasoline blended fuels based on their vapor pressure.

1.		REFERENCES

1.1		ASTM D 323.  “Standard Test Method for Vapor Pressure of Petroleum Products (Reid Method). “

1.2		CBPL 27-26/ASTM D-4814.  ”Standard Specification for Automotive Spark-Ignition Engine Fuel.”

END