U.S. CUSTOMS AND BORDER PROTECTION

LABORATORY METHODS

CBPL METHOD 39-27

Guidelines for the Identification of HTSUS Section VII Materials

# SAFETY STATEMENT

This CBPL Method cannot fully address safety issues that may arise from its use.  The analyst is responsible for assessing potential safety issues associated with a given method at its point of use.

Before using this method, the analyst will consider all general laboratory safety precautions.  In particular, the analyst will identify and implement suitable health and safety measures and will comply with all pertinent regulations.

METHOD UNCERTAINTY

The uncertainty of measurement for this method is specific to each laboratory.

0.		SCOPE AND FIELD OF 					APPLICATION

This method is designed to aid in the identification of polymeric materials provided for in Chapter 39 of the Harmonized Tariff Schedule of the United States (HTSUS).

1.		REFERENCES

This list is provided for general guidance and should not be considered exhaustive.  The user is expected to seek current references pertaining to this method.

1.1		AATCC Technical Manual.  American Association of Textile Chemists and Colorists.  Research Triangle Park, NC.

1.2		Bibawy, N. A.  “Preparation of Polymer Samples for Qualitative Analysis.”  Customs Laboratory Bulletin.  6(4).  1994.  Pp 187-207.

1.3		Brezinski, D.R. (Editor).  An Infrared Spectroscopy Atlas for the Coatings Industry.  Federation of Societies for Coating Technology.  Blue Bell, PA.  1991.

1.4		Byington, J., Mohl, D., and  Chan, E.  “Analysis of Plastic Sheeting by FT-IR Microscopy.”  Customs Laboratory Bulletin.  6(2).  1994.  Pp. 115-123.

1.5		Crompton, T.R.  Analysis of Polymers:  An Introduction.  Pergamon Press.  Oxford.  1989.

1.6		Farber, Leon.  “Polymer Types and Structures.”  Customs Laboratory Bulletin.  6(2).  1994.  Pp 53-84.

1.7		Hahn, E.C.  “A Review of the Quantitative Analysis of Soluble Polymers Using Proton Nuclear Magnetic Resonance Spectroscopy.”  Customs Laboratory Bulletin.  6(2).  1994.  Pp 89-105.

1.8		Harris, R.  “Qualitative Identification Tests for Plastics.”  Customs Laboratory Bulletin.  6(4).  1994.  Pp 209-225.

1.9		Hasla, J., Willis H.A., and Squirrell, D.C.M.  Identification and Analysis of Plastics, Second Edition.  Butterworths.  London.  1972.

1.10		Hummel, D.O., and Scholl, F.  Atlas of Polymer and Plastics Analysis, Second Edition.  Hanser.  Munich.  1981.

1.11		Krause, A., Lange, A., and Ezrin, M.  Plastics Analysis Guide:  Chemical and Instrumental Analysis.  Hanser.  Munich  1983.

1.12		Lucchesi, C.A., and McGuinness, J.D.  “Methods for the Analysis of Plastic Resins,” in the Handbook of Analytical Chemistry, L. Meites (Editor).  McGraw-Hill.  New York.  1963.  Pp 13-210 to 13-226.

1.13		Mark, H.F., Bikales, N.M., Overberger, C.G., Menges, B., and Kroschwitz, J.I. (Editors).  Encyclopedia of Polymer Science and Technology, Second Edition.  John Wiley and Sons.  New York.  NEED DATE

1.14		Pham, Q.T., Petiaud, R., Waton, H., and Llamo-Darricades, M.F.  Proton and Carbon NMR Spectra of Polymers.  CRC Press.  Boca Raton, FL.  1991.

1.15		Roff, W.J., Scott J.R., and Pacitti., J.  Handbook of Common Polymers.  CRC Press.  Cleveland, Ohio.  1991.

1.16		 Simons, W.W. and Zanger, M.  The Sadtler Guide to the NMR Spectra of Polymers.  Sadtler Research Laboratories.  Philadelphia.  1973.

1.17		Wagner, W.J.  “A Tentative Method for Distinguishing Low Density Polyethylene from Linear Low Density Polyethylene.”  Customs Laboratory Bulletin.  6(2).  1994.  Pp 85-88.

1.18		Wiberly, J.S.  Chapter 5, “The Analysis of Textile Structure.”  Analytical Methods for a Textile Laboratory, J.W. Weaver (Editor).  American Association of Textile Chemists and Colorists.  Research Triangle Park, NC.  1984.

END